| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
| 1 | Installation & alignment of take-up carriage & guides is in accordance with the drawings & survey report attached | ☐ | ☐ | ☐ |  |  |  |
| 2 | Platforms, handrail & gates installed as per relevant drawings | ☐ | ☐ | ☐ |  |  |  |
| 3 | Take-up mass corresponds to the specifications on the drawings & is clearly marked on the take- up | ☐ | ☐ | ☐ | - Mass: _______________ T |  |  |
| 4 | Asset number in 50mm black lettering on a yellow background painted on both counterweight & safety chain system | ☐ | ☐ | ☐ |  |  |  |
| 5 | All bolted connections checked for correct tension – Bolt FIC Completed | ☐ | ☐ | ☐ |  |  |  |
| 6 | Visually check for any possible obstructions that may limit freedom of take- up movement | ☐ | ☐ | ☐ |  |  |  |
| 7 | Winch, rope, sheaves installed | ☐ | ☐ | ☐ |  |  |  |
| 8 | Take-up is in correct position with belt tensioned | ☐ | ☐ | ☐ |  |  |  |
| 9 | Remote grease lines purged, charged, secure & manifold is easily accessible outside guarding | ☐ | ☐ | ☐ |  |  |  |
| 10 | Flexible lube line loop sufficient for full take up travel | ☐ | ☐ | ☐ |  |  |  |
| 11 | Counterweight lifting system installed as per drawings | ☐ | ☐ | ☐ |  |  |  |
| 12 | Ensure all rigging equipment e.g. shackles, wedge sockets, chains, snatch blocks etc are rated for the applicable load & installed/ secured correctly | ☐ | ☐ | ☐ |  |  |  |
| 13 | Maintenance chains securely attached in the stowed position | ☐ | ☐ | ☐ |  |  |  |
| 14 | Snatch block sheaves greased | ☐ | ☐ | ☐ |  |  |  |
| 15 | Ladder with safety system installed correctly | ☐ | ☐ | ☐ |  |  |  |
| 16 | Shedder plate installed correctly & rubbers secured to plates | ☐ | ☐ | ☐ |  |  |  |
| 17 | Guarding installed | ☐ | ☐ | ☐ |  |  |  |
| 18 | Signage installed as per client specifications | ☐ | ☐ | ☐ |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |