| SPECIFICATIONS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Manufacturer |  | Model |  | Serial Number |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
|  | GENERAL |  |  |  |  |  |  |
| 1 | Metal detector installed as per Drawings | ☐ | ☐ | ☐ |  |  |  |
| 2 | Metal detector aligned square to belt | ☐ | ☐ | ☐ |  |  |  |
| 3 | All bolts have been tensioned correctly | ☐ | ☐ | ☐ |  |  |  |
| 4 | Transmitter coil swings in direction of belt travel | ☐ | ☐ | ☐ |  |  |  |
| 5 | Transmitter coil mounted at a height to clear normal conveyor loads | ☐ | ☐ | ☐ |  |  |  |
| 6 | Receiver coil is mounted to the correct distance below belt line | ☐ | ☐ | ☐ | Distance: ___________ mm |  |  |
| 7 | Coils mounted across conveyor on centreline between two idlers | ☐ | ☐ | ☐ |  |  |  |
| 8 | Idler spacing’s at coil location is as per vendor specification or non metallic idlers used | ☐ | ☐ | ☐ |  |  |  |
| 9 | Coil support structure is located so the conveyor will stop before any tramp metal is discharged | ☐ | ☐ | ☐ |  |  |  |
| 10 | Control unit is located in a vibration free area at the correct distance from the coils | ☐ | ☐ | ☐ |  |  |  |
| 11 | Tramp metal marker correctly aligned and secured &/ or beacon is located so it is clearly visible | ☐ | ☐ | ☐ |  |  |  |
| 12 | Clearances from metal skirting, chutes, magnets, dust covers etc are as per vendor specifications | ☐ | ☐ | ☐ |  |  |  |
| 13 | All signage installed as per client specification | ☐ | ☐ | ☐ |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |