| - FEEDER SPECIFICATIONS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Manufacturer |  | - Model |  | - Serial Number |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
|  | FEEDER |  |  |  |  |  |  |
| 1 | Grizzly installed in accordance with the drawings & manufacturers instructions | ☐ | ☐ | ☐ |  |  |  |
| 2 | Isolation frame installed in accordance with the drawings & manufacturers instructions | ☐ | ☐ | ☐ |  |  |  |
| 3 | Check for any mechanical damage | ☐ | ☐ | ☐ |  |  |  |
| 4 | Grizzly bar retension bolts checked for tension after transit. Bolt FIC attached | ☐ | ☐ | ☐ |  |  |  |
| 5 | All spring centre measurements checked to drawing specifications | ☐ | ☐ | ☐ |  |  |  |
| 6 | Spring base plate centres are square – diagonals | ☐ | ☐ | ☐ |  |  |  |
| 7 | Foundation for spring spigot base plates within manufacturer specification IOM Specification: | ☐ | ☐ | ☐ |  |  |  |
| 8 | Spring spigot base plates level within within manufacturer specification longitudinally IOM Specification: | ☐ | ☐ | ☐ |  |  |  |
| 9 | Spring spigot base plates level within manufacturer spec transversely IOM Specification: | ☐ | ☐ | ☐ |  |  |  |
| 10 | All spring base plate bolts have been tensioned in accordance with the manufacturers instructions | ☐ | ☐ | ☐ |  |  |  |
| 11 | All bolts installed with self-locking nuts | ☐ | ☐ | ☐ |  |  |  |
| 12 | Grizzly transverse alignment is within manufacturer specification at either side wall | ☐ | ☐ | ☐ |  |  |  |
| 13 | Dust curtains installed correctly | ☐ | ☐ | ☐ |  |  |  |
| 14 | All guarding installed | ☐ | ☐ | ☐ |  |  |  |
| 15 | All signage installed as per client specification | ☐ | ☐ | ☐ |  |  |  |

| SPRING HEIGHTS (measure and record spring length at centre of spring pack, length must be equal within 4mm side to side) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Flow |  |  |  |  |  |  |
|  | Feeder | Isolation Frame |  |  | Feeder | Isolation Frame |
| LHD | mm | mm |  | RHD | mm | mm |
| LHF | mm | mm |  | RHF | mm | mm |

| CLEARANCES |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Clearances on Structure- general |  | mm |  |  |  |  |  |
| Feed Chute Clearance |  | Side L/H: |  | mm | Side R/H: |  | mm |
|  |  | Back L/H: |  | mm | Back R/H: |  | mm |
|  |  | Deck L/H: |  | mm | Deck R/H: |  | mm |
| Discharge Chute Clearance | L/H Top: |  | mm |  | R/H Top: | mm |  |
|  | L/H Bot: |  | mm |  | R/H Bot: | mm |  |
|  | L/H Side: |  | mm |  | R/H Side: | mm |  |

| - EXCITER AND DRIVE UNIT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Manufacturer |  | - Model |  | - Serial Number |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
|  | EXCITER & DRIVE UNIT |  |  |  |  |  |  |
| 16 | Motor hold down bolts tensioned correctly. Bolt Torque FIC completed | ☐ | ☐ | ☐ |  |  |  |
| 17 | Layshaft bearing hold down bolts tensioned correctly. Bolt Torque FIC completed | ☐ | ☐ | ☐ |  |  |  |
| 18 | Exciter transport protection removed | ☐ | ☐ | ☐ |  |  |  |
| 19 | Exciter Bolts Tensioned Correctly- required to check tension after transport. Bolt Torque FIC completed | ☐ | ☐ | ☐ | Torque: ___________ Nm |  |  |
| 20 | Cardan Shaft (drive) coupling bolts tensioned correctly. Bolt Torque FIC completed | ☐ | ☐ | ☐ |  |  |  |
| 21 | Cardan Shaft (timing) coupling bolts tensioned correctly. Bolt Torque FIC completed | ☐ | ☐ | ☐ |  |  |  |
| 22 | Exciters aligned in phase. Alignment Report attached | ☐ | ☐ | ☐ |  |  |  |
| 23 | Liners installed and tensioned where applicable. Bolt FIC completed | ☐ | ☐ | ☐ |  |  |  |
| 24 | All guarding installed correctly | ☐ | ☐ | ☐ |  |  |  |
| 25 | FIC-0022 V-belt Drive check sheet completed & attached | ☐ | ☐ | ☐ |  |  |  |

| LUBRICATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Oil level in Exciters (measure using dipstick) | L |  | Motor Bearings checked for grease | Y | N |
| Oil type: |  |  | Grease type: |  |  |
| Oil Qty: |  |  | Grease Qty |  |  |
| Drive Shaft Spline & uni joints greased | Y | N |  |  |  |
| Grease type: |  |  |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |