If units are vendor supplied fully coupled & aligned, then mark items as “Vendor” & attach appropriate Vendor Factory Test Sheets.

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
| 1 | Laser alignment of shafts completed & report attached | ☐ | ☐ | ☐ |  |  |  |
| 2 | Flexible couplings installed & appropriate check sheets attached | ☐ | ☐ | ☐ |  |  |  |
| 3 | Ensure fluid coupling is securely fitted | ☐ | ☐ | ☐ |  |  |  |
| 4 | No leakage evident from body or shaft seals | ☐ | ☐ | ☐ |  |  |  |
| 5 | Coupling is free to rotate and driver is free from driven | ☐ | ☐ | ☐ |  |  |  |
| 6 | Switching element (proximity sensor) is correctly installed and adjusted relative to the initiator (receiver) | ☐ | ☐ | ☐ |  |  |  |
| 7 | Guarding installed, clear of rotating elements & adequate to protect personnel in the event of a plug rupture | ☐ | ☐ | ☐ |  |  |  |
| 8 | Correct fusible plug(s) installed | ☐ | ☐ | ☐ | Rupture Temperature: _______ °C |  |  |
| 9 | Coupling is filled with oil to correct level | ☐ | ☐ | ☐ | Oil Type: ______________ Oil Qty: ______________ |  |  |
| 10 | Fill plug angle (referenced from TDC) | ☐ | ☐ | ☐ | Angle: _______________° |  |  |
| 11 | All signage installed as per client specification | ☐ | ☐ | ☐ |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |