| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
| 1 | Pulley carriage and rails are installed and aligned in accordance with the drawings & survey report attached | ☐ | ☐ | ☐ |  |  |  |
| 2 | Take up tower installation & alignment of carriage & guides is in accordance with the drawings & survey report attached | ☐ | ☐ | ☐ |  |  |  |
| 3 | Winch Rope, Sheaves installed | ☐ | ☐ | ☐ |  |  |  |
| 4 | Take-up mass corresponds to the specifications on the drawings & is clearly marked on the take- up | ☐ | ☐ | ☐ | Mass: ______________ T |  |  |
| 5 | Asset number in 50mm black lettering on a yellow background painted on both counterweight & safety chain system | ☐ | ☐ | ☐ |  |  |  |
| 6 | All bolted connections checked for correct tension. Bolt FIC Completed | ☐ | ☐ | ☐ |  |  |  |
| 7 | Visually check for any possible obstructions that may limit freedom of take- up movement on both take up pulley carriage rails & take up tower | ☐ | ☐ | ☐ |  |  |  |
| 8 | Rotate sheaves by hand to check freedom of rotation | ☐ | ☐ | ☐ |  |  |  |
| 9 | Correct diameter wire rope used as per drawings | ☐ | ☐ | ☐ |  |  |  |
| 10 | Take– up in correct position with belt tensioned | ☐ | ☐ | ☐ |  |  |  |
| 11 | Wire rope installed correctly & seats within the sheave groove | ☐ | ☐ | ☐ |  |  |  |
| 12 | Remote grease lines purged, charged, secure & manifold is easily accessible outside guarding | ☐ | ☐ | ☐ | Grease Type: ______________ |  |  |
| 13 | All sheaves greased | ☐ | ☐ | ☐ |  |  |  |
| 14 | Keeper brackets have correct clearance under rail support beams | ☐ | ☐ | ☐ |  |  |  |
| 15 | Carriage wheels seated on rail correctly | ☐ | ☐ | ☐ |  |  |  |
| 16 | Wire rope restraints securely fastened to structure | ☐ | ☐ | ☐ |  |  |  |
| 17 | Counterweight lifting system installed as per drawings | ☐ | ☐ | ☐ |  |  |  |
| 18 | Ensure all rigging equipment e.g. shackles, wedge sockets etc are rated for the applicable load & installed/ secured correctly | ☐ | ☐ | ☐ |  |  |  |
| 19 | Maintenance chains securely attached in the stowed position |  |  |  |  |  |  |
| 20 | Snatch block sheaves greased |  |  |  |  |  |  |
| 21 | Ladder with “Lad-Saf” safety system installed correctly |  |  |  |  |  |  |
| 22 | Shedder plate installed correctly & rubbers secured to plates |  |  |  |  |  |  |
| 23 | Guarding installed |  |  |  |  |  |  |
| 24 | Signage installed as per client specification |  |  |  |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |