| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
| 1 | Check installation against drawings | ☐ | ☐ | ☐ |  |  |  |
| 2 | Thorough visual inspection undertaken for any transport damage including lubrication components | ☐ | ☐ | ☐ |  |  |  |
| 3 | Apron Feeder alignment/ survey report attached | ☐ | ☐ | ☐ |  |  |  |
| 4 | All bearing transport protection removed | ☐ | ☐ | ☐ |  |  |  |
| 5 | Pan bolt tension checked as per value indicated in vendor IOM | ☐ | ☐ | ☐ |  |  |  |
| 6 | Check idler bolts. If any found loose, idler alignment check to be undertaken | ☐ | ☐ | ☐ |  |  |  |
| 7 | All remaining bolted connections checked for correct tension. QU-FIC-0053 & QU-FIC-0075 attached as applicable | ☐ | ☐ | ☐ |  |  |  |
| 8 | Check all hoses & cables are secured clear from rotating elements | ☐ | ☐ | ☐ |  |  |  |
| 9 | Check clearance between flights & skirts as per vendor IOM | ☐ | ☐ | ☐ |  |  |  |
| 10 | Check all rotating elements are free from obstruction | ☐ | ☐ | ☐ |  |  |  |
| 11 | Drive / Coupling Alignment carried out and applicable FIC attached | ☐ | ☐ | ☐ |  |  |  |
| 12 | All lubrication lines have been connected purged & lube tank filled with grease | ☐ | ☐ | ☐ | Grease Type: ____________ Grease Qty: ____________ |  |  |
| 13 | Check for any oil leaks from track rollers, return rollers & track pins | ☐ | ☐ | ☐ |  |  |  |
| 14 | Check chain tension as per vendor IOM | ☐ | ☐ | ☐ |  |  |  |
| 15 | Air blast cooler oil lines connected & fans rotate freely | ☐ | ☐ | ☐ |  |  |  |
| 16 | Shedder plates installed & bolts tensioned | ☐ | ☐ | ☐ |  |  |  |
| 17 | Hydraulic Motor & Power Pack installed & check sheet FIC-0052 attached | ☐ | ☐ | ☐ |  |  |  |
| 18 | All guarding in place & secure | ☐ | ☐ | ☐ |  |  |  |
| 19 | Signage in place as per Client Specification | ☐ | ☐ | ☐ |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |