| SPECIFICATIONS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Manufacturer |  | Model |  | Serial Number |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
| 1 | Cutter is installed as per the manufacturer’s instruction | ☐ | ☐ | ☐ |  |  |  |
| 2 | Cutter is secure and positioned as per the drawings/ model | ☐ | ☐ | ☐ |  |  |  |
| 3 | All temporary bracing removed | ☐ | ☐ | ☐ |  |  |  |
| 4 | Cutter hold Down bolts tensioned to manufacturers specification. Bolt FIC Completed | ☐ | ☐ | ☐ | Torque: _______________ Nm |  |  |
| 5 | Cutter drive unit is attached securely to the chute and all bolts are tensioned | ☐ | ☐ | ☐ |  |  |  |
| 6 | Non drive end bearings have been greased | ☐ | ☐ | ☐ | Grease Type: _______________ |  |  |
| 7 | Gearbox filled with oil/ levels checked | ☐ | ☐ | ☐ | Oil Type: ____________________ Oil Quantity: _________________ |  |  |
| 8 | Belt ends clamped correctly | ☐ | ☐ | ☐ |  |  |  |
| 9 | Excess belt trimmed manufacturer / client specification past end of clamp | ☐ | ☐ | ☐ |  |  |  |
| 10 | Belt tension checked- at mid span, belt deflects as per manufacturer / client specification | ☐ | ☐ | ☐ |  |  |  |
| 11 | Belt support rollers adjusted to keep belt clear of pulley lower flanges | ☐ | ☐ | ☐ |  |  |  |
| 12 | Belt teeth mesh correctly with gear drives | ☐ | ☐ | ☐ |  |  |  |
| 13 | Rail scraper blocks installed correctly | ☐ | ☐ | ☐ |  |  |  |
| DISENGAGE DRIVE CLUTCH & UNDERTAKE THE FOLLOWING TESTS BY MOVING CARRIAGE BY HAND. ENSURE ALL SAFETY PRECAUTIONS ARE IN PLACE. |  |  |  |  |  |  |  |
| 13 | Carriage wheels in full contact with both faces of both rails over full stroke |  |  |  |  |  |  |
| 14 | Wheels ride centrally on the rails & don’t lift off or stop turning |  |  |  |  |  |  |
| 15 | Carriage alignment rollers checked clearance tolerance per manufacturer / client specification to the underside of the side pressings |  |  |  |  |  |  |
| 16 | Thrust wheel adjusted as not to impose a preload in the opposite direction & runs smoothly along length of rail |  |  |  |  |  |  |
| 17 | Cutter discharge correctly aligned with sample chute & does not clash during manual stroking of unit |  |  |  |  |  |  |
| 18 | All guarding installed & secure |  |  |  |  |  |  |
| 19 | All signage installed as per client specification |  |  |  |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |