All travel wheel, rack chain & shuttle framework shall be surveyed & passed for squareness & parallelism before transport to site. If survey documentation cannot be witnessed or if a series of bolts are found to be loose, then an on-site survey is to be undertaken & results attached to this check sheet.

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
| 1 | Shuttle has been examined for any structural transport damage | ☐ | ☐ | ☐ |  |  |  |
| 2 | Pinions & drive shaft checked for any transport damage | ☐ | ☐ | ☐ |  |  |  |
| 3 | All structural bolted connections to be 10% proof checked (minimum). Bolt tension QU-FIC-0075 attached | ☐ | ☐ | ☐ |  |  |  |
| 4 | Rack chain has 4 bolts installed on every second link – 10% proof checked (minimum) | ☐ | ☐ | ☐ |  |  |  |
| 5 | All rail fasteners tensioned & marked – 10 % proof checks completed (minimum). Bolt torque QU-FIC-0053 attached | ☐ | ☐ | ☐ |  |  |  |
| 6 | Rail alignment & shuttle axle survey has been undertaken, within specification & survey report attached to check sheet | ☐ | ☐ | ☐ |  |  |  |
| 7 | End stops and buffers are installed in accordance with the drawings and manufacturer’s instructions | ☐ | ☐ | ☐ |  |  |  |
| 8 | Shuttle grease manifold lines purged, charged with grease & lines checked for leakage | ☐ | ☐ | ☐ | Grease Type: ___________ Grease Qty: ___________ |  |  |
| 9 | All bearings have grease present at labyrinth seals | ☐ | ☐ | ☐ |  |  |  |
| 10 | Drive sprocket alignment with chain confirmed | ☐ | ☐ | ☐ |  |  |  |
| 11 | Chain split link at shuttle pivot point matches actual chain pitch | ☐ | ☐ | ☐ |  |  |  |
| 12 | Shuttle pivot pins installed with flange plate bolts tensioned correctly | ☐ | ☐ | ☐ |  |  |  |
| 13 | Shuttle pivot pins greased | ☐ | ☐ | ☐ |  |  |  |
| 14 | Rail scraper blocks installed | ☐ | ☐ | ☐ |  |  |  |
| 15 | Intermediate Stop Lever is easily actuated & locking pins can be installed in both open & closed position | ☐ | ☐ | ☐ |  |  |  |
| 16 | Intermediate Stop Lever linkages & pillow blocks greased | ☐ | ☐ | ☐ |  |  |  |
| 17 | Hold down rollers set at 18mm (nominal) below underside of rail support beam flange | ☐ | ☐ | ☐ |  |  |  |
| 18 | Hold Down Rollers jacking bolts have lock nuts tensioned | ☐ | ☐ | ☐ |  |  |  |
| 19 | All travel wheels have contact with the rail | ☐ | ☐ | ☐ |  |  |  |
| 20 | Sprocket gauges are attached to drive frame with suitable chain | ☐ | ☐ | ☐ |  |  |  |
| 21 | All guards are installed, secure and clear of the travel path of the shuttle/tripper: | ☐ | ☐ | ☐ |  |  |  |
| 22 | Full travel zone has been checked and is clear of any obstructions in the path of the shuttle/tripper | ☐ | ☐ | ☐ |  |  |  |
| 23 | A test installation for the Maintenance Recovery Pins has been undertaken | ☐ | ☐ | ☐ |  |  |  |
| 24 | Shuttle manually “driven” (No Power) into maintenance zone & locking pins engaged/ disengaged | ☐ | ☐ | ☐ |  |  |  |
| 25 | All signage installed as per client specification | ☐ | ☐ | ☐ |  |  |  |
| 26 | QU-FIC-0003 Drives check sheet completed & attached | ☐ | ☐ | ☐ |  |  |  |
| 27 | QU-FIC-0005 Hydraulic Disc Brakes check sheet completed & attached | ☐ | ☐ | ☐ |  |  |  |
| 28 | QU-FIC-0020 Idlers check sheet completed & attached | ☐ | ☐ | ☐ |  |  |  |
| 29 | QU-FIC-0021 Pulleys check sheet completed & attached | ☐ | ☐ | ☐ |  |  |  |
| 30 | QU-FIC-0018 Belt Scraper check sheet completed & attached | ☐ | ☐ | ☐ |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |