| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
|  | PRE-PLACEMENT INSPECTION |  |  |  |  |  |  |
| 1 | Verify equipment has been aligned and levelled Attached survey report identifying item(s) to be grouted | ☐ | ☐ | ☐ |  |  |  |
| 2 | Surface is prepared for grout (scabbled, water conditioned etc) | ☐ | ☐ | ☐ |  |  |  |
| 3 | Sleeves are clean and clear. | ☐ | ☐ | ☐ |  |  |  |
| 4 | Displaced air can be vented | ☐ | ☐ | ☐ |  |  |  |
| 5 | Formwork is properly waxed. | ☐ | ☐ | ☐ |  |  |  |
| 6 | Formwork is sealed. | ☐ | ☐ | ☐ |  |  |  |
| 7 | Grout product approved for use | ☐ | ☐ | ☐ |  |  |  |
| 8 | Curing products/facilities available and approved for use | ☐ | ☐ | ☐ |  |  |  |
| 9 | Facilities for airing available | ☐ | ☐ | ☐ |  |  |  |
|  | MIXING & FINISHING INSPECTION |  |  |  |  |  |  |
| 10 | Grout batch certificates available | ☐ | ☐ | ☐ |  |  |  |
| 11 | Grout mixed in accordance with manufacturer recommendations, method statement complete with daily record sheet | ☐ | ☐ | ☐ |  |  |  |
| 12 | Ambient temperature in accordance with specification | ☐ | ☐ | ☐ |  |  |  |
| 13 | Grout temperature in accordance with specification | ☐ | ☐ | ☐ |  |  |  |
| 14 | Test cube samples taken where applicable and results compliant. 3 Test cubes each day per grout type used. | ☐ | ☐ | ☐ |  |  |  |
| 15 | Grouting complete and protective coating applied where applicable. | ☐ | ☐ | ☐ |  |  |  |
| 16 | Grouting inspected with formwork removed | ☐ | ☐ | ☐ |  |  |  |
| 17 | Chamfers Acceptable | ☐ | ☐ | ☐ |  |  |  |
| 18 | Grout in full contact and crack free | ☐ | ☐ | ☐ |  |  |  |
| 19 | Base plate hammer sound tested for voids, grout visible in breather holes. | ☐ | ☐ | ☐ |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |