| SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| Manufacturer |  | Model |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
|  | GENERAL |  |  |  |  |  |  |
| 1 | Installation as per manufacturers instructions | ☐ | ☐ | ☐ |  |  |  |
| 2 | All hold down bolts tensioned correctly on inlet & outlet ducting | ☐ | ☐ | ☐ |  |  |  |
| 3 | All flanged connections assembled using approved sealant | ☐ | ☐ | ☐ |  |  |  |
| 4 | All ducting supports installed correctly | ☐ | ☐ | ☐ |  |  |  |
| 5 | Ducting adjustable gates move freely | ☐ | ☐ | ☐ |  |  |  |
| 6 | All ducting clear of foreign objects | ☐ | ☐ | ☐ |  |  |  |
| 7 | Ducting Balance gates open where applicable | ☐ | ☐ | ☐ |  |  |  |
| 8 | Inspection doors on filter unit & ducting open/ close freely & seal tightly | ☐ | ☐ | ☐ |  |  |  |
| 9 | Filter bag cuffs seated correctly | ☐ | ☐ | ☐ |  |  |  |
| 10 | Filter bag mouthpiece prongs correctly inserted in bag frames | ☐ | ☐ | ☐ |  |  |  |
| 11 | All locking latches engaged | ☐ | ☐ | ☐ |  |  |  |
| 12 | All paintwork has been touched up | ☐ | ☐ | ☐ |  |  |  |
| 13 | All debris removed from hoppers | ☐ | ☐ | ☐ |  |  |  |
| 14 | Hopper access doors securely bolted shut | ☐ | ☐ | ☐ |  |  |  |
|  | EXHAUST FAN & DRIVE |  |  |  |  |  |  |
| 15 | Transit clamps removed | ☐ | ☐ | ☐ |  |  |  |
| 16 | All hold down bolts tensioned correctly. Bolting FIC Attached | ☐ | ☐ | ☐ |  |  |  |
| 17 | Exhaust fan impellor rotated by hand to ensure free rotation | ☐ | ☐ | ☐ |  |  |  |
| 18 | Fan to casing clearance within manufacturer / IOM Specification | ☐ | ☐ | ☐ |  |  |  |
| 19 | Shaft Bearings greased as per manufacturer IOM, Seal clearance Checked | ☐ | ☐ | ☐ | Seal Clearance: ____________ |  |  |
| 20 | Fan casing free of debris | ☐ | ☐ | ☐ |  |  |  |
| 21 | Fan dampers have free movement over full quarter of a turn without rubbing on casing | ☐ | ☐ | ☐ |  |  |  |
| 22 | Coupling on exhaust fan is decoupled awaiting direction test- Flexible Coupling check sheet FIC-0056 attached | ☐ | ☐ | ☐ |  |  |  |
| 23 | All guarding installed | ☐ | ☐ | ☐ |  |  |  |
| 24 | All signage installed as per client specification | ☐ | ☐ | ☐ |  |  |  |
|  | ROTARY VALVES & DISCHARGE CHUTE |  |  |  |  |  |  |
| 25 | Bolt tensions checked either side of valve | ☐ | ☐ | ☐ |  |  |  |
| 26 | All debris has been removed | ☐ | ☐ | ☐ |  |  |  |
| 27 | Rotary valve drive chain tensioned correctly | ☐ | ☐ | ☐ |  |  |  |
| 28 | Discharge chute sealed against dust hood of conveyor | ☐ | ☐ | ☐ |  |  |  |
|  | BAG CLEANING TRAVELLER (where applicable) |  |  |  |  |  |  |
| 29 | Thrust wheels seated correctly on traversing rail | ☐ | ☐ | ☐ |  |  |  |
| 30 | Bag cleaning traveller rails free from obstructions | ☐ | ☐ | ☐ |  |  |  |
| 31 | Bag cleaning traveller drive chain/ coupling/ v belt tensioned correctly | ☐ | ☐ | ☐ |  |  |  |
| 32 | Lip seals positioned correctly | ☐ | ☐ | ☐ |  |  |  |
|  | COMPRESSOR (where applicable) |  |  |  |  |  |  |
| 33 | Installed as per manufacturers specification | ☐ | ☐ | ☐ |  |  |  |
| 34 | All lines are free from damage & connections are tight | ☐ | ☐ | ☐ |  |  |  |
| 35 | Safety relief valves set at correct pressure | ☐ | ☐ | ☐ | Pressure: _____________ kPa |  |  |

| LUBRICATION (If any item pre lubricated at factory stage, mark as vendor completed) |  |  |  |
| --- | --- | --- | --- |
| Item | Manufacturer | Type | Amount |
| Exhaust Fan Bearings |  |  |  |
| Rotary Valve Gearbox Oil |  |  |  |
| Rotary Valve Bearings |  |  |  |
| Traveler Drive Gearbox |  |  |  |
| Compressor |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |