| SPECIFICATIONS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MOTOR |  | POWER PACK |  |  |  |
| Manufacturer |  | Manufacturer |  |  |  |
| Model |  | Model |  |  |  |
| Serial Number |  | Serial Number |  |  |  |
| COOLING UNIT |  | HYDRAULIC FLUID |  |  |  |
| Manufacturer |  | Manufacturer |  |  |  |
| Model |  | Specification |  |  |  |
| Serial Number |  | Tank | L | Motor | L |
|  |  | Filter(s) | micron |  |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
|  | GENERAL |  |  |  |  |  |  |
| 1 | Installation is in accordance with the manufacturers instructions | ☐ | ☐ | ☐ |  |  |  |
| 2 | Filter elements installed, flow direction correct & inspected before first fill | ☐ | ☐ | ☐ |  |  |  |
| 3 | Filters installed with correct micron rating | ☐ | ☐ | ☐ |  |  |  |
| 4 | Reservoir breather installed | ☐ | ☐ | ☐ |  |  |  |
| 5 | All hydraulic hoses not stressed or twisted during installation | ☐ | ☐ | ☐ |  |  |  |
| 6 | Hydraulic tank cleaned | ☐ | ☐ | ☐ |  |  |  |
| 7 | Hydraulic tank filled to level marker on site gauge | ☐ | ☐ | ☐ |  |  |  |
| 8 | Power pack cabinet located to allow specified clearance on all sides | ☐ | ☐ | ☐ |  |  |  |
| 9 | Motor drain lines connected to hydraulic tank with minimum restrictions | ☐ | ☐ | ☐ |  |  |  |
| 10 | Uppermost vent plug removed from pump & pre- filled with oil via drain plug until all air is purged | ☐ | ☐ | ☐ |  |  |  |
| 11 | All hydraulic connections tightened | ☐ | ☐ | ☐ |  |  |  |
| 12 | Power pack & cooling unit hold down bolts are tensioned correctly. Bolt FIC completed | ☐ | ☐ | ☐ |  |  |  |
| 13 | Cooling unit oil lines connected & fans rotate freely by hand | ☐ | ☐ | ☐ |  |  |  |
| 14 | All signage installed | ☐ | ☐ | ☐ |  |  |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
|  | HYDRAULIC MOTOR |  |  |  |  |  |  |
| 15 | Motor installed on shaft as per manufacturers instruction | ☐ | ☐ | ☐ |  |  |  |
| 16 | Motor mounting shaft thoroughly cleaned of Molycote before mounting motor | ☐ | ☐ | ☐ | Bolt Torque: ___________ Nm |  |  |
| 17 | Motor rotation is such that the two drain ports are positioned vertically | ☐ | ☐ | ☐ |  |  |  |
| 18 | Torque arm fitted to motor & bolts tensioned correctly | ☐ | ☐ | ☐ |  |  |  |
| 19 | Exciter bearings greased | ☐ | ☐ | ☐ | Brand: __________________ Type: __________________ |  |  |
| 20 | Torque arm correctly secured to attachment arms & reaction point | ☐ | ☐ | ☐ | Torque: _________________ Nm |  |  |
| 21 | Motor hydraulic lines connected to give correct direction of rotation | ☐ | ☐ | ☐ |  |  |  |
| 22 | Check all hydraulic connections and couplings are tight | ☐ | ☐ | ☐ |  |  |  |
| 23 | Spline cavity filled with oil, plug re- tensioned & no oil leaks from rear o- ring seal | ☐ | ☐ | ☐ | Plug Torque: ______________ Nm |  |  |
| 24 | Motor filled thru uppermost plug in outer front casing. | ☐ | ☐ | ☐ | Fill filter micron rating: |  |  |
| 25 | Motor drain line connected | ☐ | ☐ | ☐ |  |  |  |
| 26 | All guarding installed & secure | ☐ | ☐ | ☐ |  |  |  |
| 27 | All signage installed as per client specification | ☐ | ☐ | ☐ |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |