| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
| 1 | All hydraulic piping & fittings in accordance with client specification | ☐ | ☐ | ☐ |  |  |  |
| 2 | All hydraulic lines are clear of pulley & belt | ☐ | ☐ | ☐ |  |  |  |
| 3 | Hydraulic hand pump oil level is correct | ☐ | ☐ | ☐ | Type: ___________________ Quantity: _________________ |  |  |
| 4 | Stainless steel plaque attached to conveyor frame stamped with recommended pressure | ☐ | ☐ | ☐ |  |  |  |
| 5 | Alignment of take- up pulley bearing guides is in accordance with the drawings | ☐ | ☐ | ☐ |  |  |  |
| 6 | Bearing slide rails are smooth | ☐ | ☐ | ☐ |  |  |  |
| 7 | Length of flexible hydraulic hoses allows full mechanism travel in both directions | ☐ | ☐ | ☐ |  |  |  |
| 8 | Clevis pins installed with locking collar secured | ☐ | ☐ | ☐ |  |  |  |
| 9 | Locking pins installed on link assembly & securing bolts installed | ☐ | ☐ | ☐ |  |  |  |
| 10 | All hydraulic fittings inspected for leaks when take up pressure applied | ☐ | ☐ | ☐ |  |  |  |
| 11 | Belt checked for correct tension | ☐ | ☐ | ☐ |  |  |  |
| 12 | Tensioning nut seated correctly within external cylinder & lock nut tightened | ☐ | ☐ | ☐ |  |  |  |
| 13 | Pulley is square to conveyor frame after take up tensioning is complete & hydraulic ram pressure removed | ☐ | ☐ | ☐ |  |  |  |
| 14 | After installation & tensioning, wrap thread of cylinder support housing & telescopic bearing clevis with Denso tape | ☐ | ☐ | ☐ |  |  |  |
| 15 | Guarding installed | ☐ | ☐ | ☐ |  |  |  |
| 16 | All signage installed as per client specification | ☐ | ☐ | ☐ |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |