| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| 1. GENERAL DATA |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General Arrangement Drawing |  |  |  | Specifications |  |  |  |  |  |  |
| Other Drawings |  |  |  | P&ID |  |  |  |  |  |  |
| Item | Description |  |  |  |  |  | OK | N/A | Punch |  |
|  | Confirm no physical damage or defects. |  |  |  |  |  |  |  |  |  |
|  | Eye Wash/Safety Shower has been installed as per drawings, manufacturers specifications and AS4775. |  |  |  |  |  |  |  |  |  |
|  | Installation configuration correct. |  |  |  |  |  |  |  |  |  |
|  | All flanges and gaskets installed and torqued correctly and as per specification. |  |  |  |  |  |  |  |  |  |
|  | All restriction orifices/flow restrictors installed as per manufacturers specification. |  |  |  |  |  |  |  |  |  |
|  | Valves operate correctly with no interference. Stay open valves stay open until manually closed. |  |  |  |  |  |  |  |  |  |
|  | Supports and fixings adequate on shower and adjacent pipework. |  |  |  |  |  |  |  |  |  |
|  | Shower proximity switch installed. (if applicable) |  |  |  |  |  |  |  |  |  |
|  | Filters installed in eyewash station. (if applicable) |  |  |  |  |  |  |  |  |  |
|  | Connection to unit is Potable Water or an approved flushing fluid. |  |  |  |  |  |  |  |  |  |
|  | Connection has capacity for uninterrupted flow of 15 minutes with both Eye Wash and Shower running. |  |  |  |  |  |  |  |  |  |
|  | Access to shower is clear and readily identified. |  |  |  |  |  |  |  |  |  |
|  | Piping is insulated or painted white for solar protection. |  |  |  |  |  |  |  |  |  |
|  | Green light is in place to show the location of the Eye Wash/Safety Shower. |  |  |  |  |  |  |  |  |  |
|  | Signage and labelling installed as per AS1319. |  |  |  |  |  |  |  |  |  |
|  | Thermal relief valve installed c/w drain line. |  |  |  |  |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |