	«Image:image1»
	«WorkBreakdownUp3Name»  

«DocumentTypeName»
«DocumentName» : «DocumentDescription»
	
	«Image:QRCode»




	Tag No. «Name» - «Description»

	Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName»

	Manufacturer.
	«ManufacturerName»
	Model No.
	«OEMPartNumber»

	Field to record Serial No.
	«SerialNumber»

	Drawing No.
	«DocumentList»
	Task No.
	«TaskName»

	Additional Information:
	«Comments»


	Item
	Description
	OK
	N/A
	Punch

	1 
	Confirm equipment has a tag attached in accordance with project specifications. Tag should not be obstructed and easily readable.
	
	
	

	2 
	Confirm the valve is installed according to supplier and project requirements.
	
	
	

	3 
	Confirm all gaskets are as per project requirements/ specifications.
	
	
	

	4 
	Confirm flange bolts tightened/torqued according to project specifications.
	
	
	

	5 
	Confirm the valve can be removed for service.
	
	
	

	6 
	Confirm direction of flow (if applicable).
	
	
	

	7 
	Confirm valve has been leak tested and passed leak test requirements for service (if applicable).
	
	
	

	8 
	Confirm condition of valve (external). Ensure valve is free of damage.
	
	
	

	9 
	Confirm pipe strain checked – all flange/ nozzle connections to be aligned and stress free.
	
	
	

	10 
	Confirm valve spindles are correctly fitted and secured: 
Confirm spindle has been lubricated with specified grease. 
Confirm spindle housings are free of dirt/ grit. 
Confirm grease nipples are correctly installed, free of damage and operable. 
Confirm grease nipples have protective end caps fitted.
	
	
	

	11 
	Confirm handles are fitted and secure.
	
	
	

	12 
	Confirm for ball valves and butterfly valves that provisions have been made to prevent over rotation when operating valves.
	
	
	

	13 
	Confirm gearbox has been correctly installed, sealed and secured. Confirm gearbox internals are intact and free of damage. Ensure correct type and quantity of grease has been applied. Ensure access cover is correctly refitted, sealed and secured (if applicable).
	
	
	

	14 
	Confirm operation of valve through full travel from both aspects Open/Closed.
	
	
	

	15 
	Confirm gland packing has been set correctly to allow full function of the valve without undue stress or wear.
	
	
	

	16 
	Confirm provision is made for locking open/ shut where required.
	
	
	

	17 
	Confirm position indicator is fitted. (if applicable)
	
	
	

	18 
	Confirm actuator has been correctly installed, sealed and secured onto valve. Ensure all bolts and gaskets are of correct specification (if applicable).
	
	
	

	19 
	Confirm actuator has been flushed and cleaned to correct cleanliness level.
	
	
	

	20 
	Confirm safe access to valve, that any ladders, platforms, stairways, and walkways required to access are installed and valve has been installed to meet AS-0000-P-00111.
	
	
	

	21 
	Confirm painting is complete and in accordance with project specifications.
	
	
	

	22 
	Confirm earth-bonding is in accordance with project specification requirements.
	
	
	

	23 
	Confirm insulation/ lagging is of correct type and fitted correctly as per specs (if required).
	
	
	

	24 
	Confirm rust preventive and any other preservation methods removed, if equipment to be put in service. If not being put into service, ensure that preservation requirements have been applied correctly in accordance with manufacturer’s/ project requirements and have been correctly recorded.
	
	
	


	COMMENTS, OBSERVATIONS & ATTACHMENTS

(Complete a Punchlist Form and attach if any of the checks above marked as “Punch”)

	

	COMPLETED

	I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards.

	PERFORMED BY CONTRACTOR:
	REVIEW BY SUPERVISOR/LEAD ENGINEER:
	EPCM ACCEPTED BY:

	Signature:
	
	Signature:
	
	Signature:
	

	Name:
	
	Name:
	
	Name:
	

	Title:
	
	Title:
	
	Title:
	

	Date:
	
	Date:
	
	Date:
	


Tag No: «Name»   Planned Task ID: «TaskName»
Doc Ref: «DocumentName»  Revision: «Revision»

                  © Rio Tinto 2020. All rights reserved                                                                    Page 2 of 2

