| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
| 1 | Remove all transport protection & bracing | ☐ | ☐ | ☐ |  |  |  |
| 2 | Thoroughly check Belt Reeler for any transport damage | ☐ | ☐ | ☐ |  |  |  |
| 3 | Installation is in accordance with relevant drawings | ☐ | ☐ | ☐ |  |  |  |
| 4 | - All bolted connections rechecked for correct tension after transport | ☐ | ☐ | ☐ |  |  |  |
| 5 | - All piping joints checked for tightness after transport | ☐ | ☐ | ☐ |  |  |  |
| 6 | - Rust inhibitor thoroughly cleaned from surfaces between shaft & motor | ☐ | ☐ | ☐ |  |  |  |
| 7 | - Motor installed on shaft as per manufacturers instructions/ specifications | ☐ | ☐ | ☐ |  |  |  |
| 8 | - Torque arm secured correctly to motor, attachment arm & reaction point | ☐ | ☐ | ☐ |  |  |  |
| 9 | - Motor hydraulic lines connected to give the correct rotation | ☐ | ☐ | ☐ |  |  |  |
| 10 | - Motor has been filled with hydraulic fluid | ☐ | ☐ | ☐ |  |  |  |
| 11 | - Drive pinion gear aligned with spider ring gear | ☐ | ☐ | ☐ |  |  |  |
| 12 | Hydraulic Power Pack skid installed & secured on Belt Reeler station- all hold down bolts tensioned | ☐ | ☐ | ☐ |  |  |  |
| 13 | Hydraulic reservoir filled with ISO VG100 hydraulic fluid to top of site gauge | ☐ | ☐ | ☐ |  |  |  |
| 14 | All hydraulic hoses not stressed or twisted during installation | ☐ | ☐ | ☐ |  |  |  |
| 15 | All hydraulic connections & couplings are tight | ☐ | ☐ | ☐ |  |  |  |
| 16 | Cooler fan rotates freely by hand | ☐ | ☐ | ☐ |  |  |  |
| 17 | Visually check that all rotating components are free from obstruction | ☐ | ☐ | ☐ |  |  |  |
| 18 | Main spider bearing caps are closed & locking pins installed | ☐ | ☐ | ☐ |  |  |  |
| 19 | Dead Shaft Turnover Rollers bearings have been greased | ☐ | ☐ | ☐ | Grease Type: _________________ |  |  |
| 20 | Spider gear & drive pinion have been lubricated | ☐ | ☐ | ☐ |  |  |  |
| 21 | Main spider bearings have been greased | ☐ | ☐ | ☐ | Grease Type: _________________ |  |  |
| 22 | Counterweight box has been attached to Belt Reeler, shackles installed/ tightened correctly & box filled with the appropriate amount of fines material | ☐ | ☐ | ☐ |  |  |  |
| 23 | All signage in place as per client specification | ☐ | ☐ | ☐ |  |  |  |
| 24 | All guarding in place | ☐ | ☐ | ☐ |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |