| Tag No. «Name» - «Description» |  |  |  |  |  |  | Task No. «TaskName» |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |  |  |  |  |  |  |  |
| Drawing No. |  |  | «DocumentList» |  |  |  |  |  |  |  |  |
| Additional Information: «Comments» |  |  |  |  |  |  |  |  |  |  |  |
| Item | Description |  |  |  |  |  |  | OK |  | N/A | Punch |
|  | Cleanliness of steel acceptable (dirt, dust, tags removed). |  |  |  |  |  |  |  |  |  |  |
|  | Foundation inspection and testing complete and accepted. (C1.005 complete) |  |  |  |  |  |  |  |  |  |  |
|  | Structure free from damage. |  |  |  |  |  |  |  |  |  |  |
|  | Members match and are free of distortion. |  |  |  |  |  |  |  |  |  |  |
|  | Structure complete as per design drawings and 3d models. |  |  |  |  |  |  |  |  |  |  |
|  | Check proper use of taper washers and flat washers for slotted holes where required. |  |  |  |  |  |  |  |  |  |  |
|  | Check to ensure all stiffeners are installed. |  |  |  |  |  |  |  |  |  |  |
|  | Shims installed as required and acceptable. Minimal number of shims to be used. (e.g. 20mm adjustment is shimmed with single 20mm shim, not 2 x 10mm shims) |  |  |  |  |  |  |  |  |  |  |
|  | Anchor bolts checked and accepted. |  |  |  |  |  |  |  |  |  |  |
|  | All structural bolts and bolted connections meet project specifications, checked and accepted. (S1.014 completed) |  |  |  |  |  |  |  |  |  |  |
|  | Verticality and level survey completed. |  |  |  |  |  |  |  |  |  |  |
|  | Welding & NDT of field welds completed and accepted (where required) NDT and Weld map reviewed. |  |  |  |  |  |  |  |  |  |  |
|  | Miscellaneous brackets and supports complete and acceptable. |  |  |  |  |  |  |  |  |  |  |
|  | Removal of temporary braces/guy wires completed and accepted. (after structural integrity achieved) |  |  |  |  |  |  |  |  |  |  |
|  | Protective coatings applied to suit SS-P118 and RVS-0000-SPE-S-003 and all touch-up painting completed. |  |  |  |  |  |  |  |  |  |  |
|  | Base-plates grouted and accepted. (S1.016 completed) |  |  |  |  |  |  |  |  |  |  |
|  | Verify that the earthing of steelwork has been checked against design drawings and connections are tight. |  |  |  |  |  |  |  |  |  |  |
|  | Secondary columns and beams are plumb, level and aligned |  |  |  |  |  |  |  |  |  |  |
|  | Bracing is properly installed |  |  |  |  |  |  |  |  |  |  |
|  | Gurts and Purlins are installed as per design or OEM recommendations |  |  |  |  |  |  |  |  |  |  |
|  | Roof cladding is properly secured with self-sealing roof screws and is in accordance with cyclone rating |  |  |  |  |  |  |  |  |  |  |
|  | Gutters and down pipes installed and secure |  |  |  |  |  |  |  |  |  |  |
|  | Side cladding is properly secured |  |  |  |  |  |  |  |  |  |  |
|  | Cladding is colour matched |  |  |  |  |  |  |  |  |  |  |
|  | Penetrations have sharp edges removed / protected |  |  |  |  |  |  |  |  |  |  |
|  | Roof vents, skylights, etc. installed correctly |  |  |  |  |  |  |  |  |  |  |
|  | Fire proofing has been completed |  |  |  |  |  |  |  |  |  |  |
|  | Final clean-up completed |  |  |  |  |  |  |  |  |  |  |
|  | Document Checks: Survey report attached |  |  |  |  |  |  |  |  |  |  |
|  | Document Checks: Drawings marked-up to “As Constructed’ status and issued to Document Control |  |  |  |  |  |  |  |  |  |  |
| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Item |  | Initial | N/A | Comments |  |  |  |  |  |
| 1 |  | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |  |  |  |  |  |  |
| 2 |  | All Punchlist items noted on a separate Punchlist form and submitted for. |  |  |  |  |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |