| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| PRE-INSTALLATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Description | OK | N/A | Punch |
|  | Notice of Intent submitted to the Commissioner prior to any works being undertaken. This includes the fabrication of the concrete panels. |  |  |  |
|  | Concrete Panel shop drawings available on site prior to undertaking the installation of concrete panels |  |  |  |
|  | Concrete Panel set out drawing available on site prior to undertaking the installation of concrete panels |  |  |  |
|  | Temporary support Torque Register available on site prior to Undertaking the installation of concrete panels |  |  |  |
|  | Concrete panel installation methodology approved for use |  |  |  |
|  | Engineered prop certification provided stating prop type, location and fixing requirements per concrete panel. |  |  |  |
|  | Concrete panel birthing certificates provided prior to any work being started. This includes materials handling, transport, lifting and installation. |  |  |  |
|  | Temporary support fixing certificate of compliance supplied by the vendor |  |  |  |
|  | Crane lifting study approved for use. (if applicable) |  |  |  |
|  | Panel Casting and transport sequence programmed from construction methodology |  |  |  |

| INSTALLATION |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Temporary supports installed as per engineered prop certification. Two supports per panel, Prop angles do not exceed 55deg and 65deg. Panel props state WLL. |  |  |  |
|  | Temporary supports torqued every 24hr and torque register completed. |  |  |  |
|  | Total packing under concrete panel feet is no higher than 40mm as per AS3850. |  |  |  |
|  | Strong backs installed as per engineered drawings (if applicable) |  |  |  |
|  | Grout/concrete installed between panel and footings. |  |  |  |
|  | Steel beams, suspended concrete slabs, steel plates installed. |  |  |  |
|  | Expanded polyethylene backing rod/board installed (if applicable) |  |  |  |
|  | Joint Sealant installed as per drawings (if applicable) |  |  |  |
| COMPLETION |  |  |  |  |
|  | Concrete test results attached stating design strength has been achieved |  |  |  |
|  | Survey report attached detailing final position of concrete panels |  |  |  |
| - 3 | Prior to the removal of temporary supports, - The Contractor’s manufacturer’s quality and design certification must be provided and approved by the Engineer - Written approval shall be obtained from the relevant designer. The relevant designer is the designer responsible for the stability of the structure at the time that the temporary supports are to be removed. - all CAT A & B items are completed and signed off above |  |  |  |

| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Item | Initial | N/A | Comments |
| 1 | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |
| 2 | All Punchlist items noted on a separate Punchlist form and submitted for. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |