| Tag No. «Name» - «Description» | Task No. «TaskName» |
| --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |
| Drawing No. «DocumentList» |  |
| Additional Information: «Comments» |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
| MODULE ASSEMBLY |  |  |  |  |
|  | Module identified and checked against Conveyor Profile & Data Sheets and Design Drawings. Module No.________________________________ |  |  |  |
|  | Return Idler frame appropriate for the type of module, correctly installed. |  |  |  |
|  | Leg Supports, appropriate for the type of module, correctly installed. |  |  |  |
|  | All module components free of Damage, Contamination and Distortion. |  |  |  |
|  | All module components correctly orientated and connected. |  |  |  |
|  | All idler roller shafts seated to full depth on Idler Frame Slots, and frames correctly orientated with provision for lateral adjustment. |  |  |  |
|  | All applicable fittings and module hardware installed as directed. |  |  |  |
|  | Module clearly marked with centreline and ID Tag. |  |  |  |
|  | Assembly Survey attached. |  |  |  |
|  | Protective coating of welds / drilled holes applied. |  |  |  |

| MODULE INSTALLATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Module installation requirements and Module ID identified from conveyor Profile & Data Sheets and Design Drawings |  |  |  |  |  |  |  |
|  | Appropriate sleeper /foundation for the type of module, correctly orientated and placed on the conveyor formation. |  |  |  |  |  |  |  |
|  | Foundation Sleeper Survey Reports completed and witnessed, Jacking Nuts pre-adjusted, oversize washers fitted. |  |  |  |  |  |  |  |
|  | Module installed in specified position and orientated on sleepers correctly. |  |  |  |  |  |  |  |
|  | Survey Jig, free of damage and/or distortion, correctly seated in conveyor module. |  |  |  |  |  |  |  |
|  | Jacking Lock Nuts correctly torqued and marked. |  |  |  |  |  |  |  |
|  | Holt-Down Nuts correctly torqued and marked. |  |  |  |  |  |  |  |
|  | All idler rollers are seated to a full depth on frame slots. |  |  |  |  |  |  |  |
|  | Rollers clocked, surveyed and accepted. |  |  |  |  |  |  |  |
|  | Module In-situ Survey completed and accepted. |  |  |  |  |  |  |  |
| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |  |  |  |  |
|  |  | Item | Initial | N/A | Comments |  |  |  |
| 1 |  | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |  |  |  |
| 2 |  | All Punchlist items noted on a separate Punchlist form and submitted for. |  |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |