| Tag No. «Name» - «Description» | Task No. «TaskName» |
| --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |
| Drawing No. «DocumentList» |  |
| Additional Information: «Comments» |  |

| STRUCTURAL INSTALLATION |  |  |  |  |  | OK | N/A | Punch |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Drawing specifying location of inspection and sign off marked up and attached. |  |  |  |  |  |  |  |
|  | Cleanliness of steel acceptable (dirt, dust, tags removed). |  |  |  |  |  |  |  |
|  | Foundation inspection and testing complete and accepted. (C1.005 complete) |  |  |  |  |  |  |  |
|  | Structure free from damage. |  |  |  |  |  |  |  |
|  | Members match and are free of distortion. |  |  |  |  |  |  |  |
|  | Structure complete as per design drawings and 3d models. |  |  |  |  |  |  |  |
|  | Check proper use of taper washers and flat washers for slotted holes where required. |  |  |  |  |  |  |  |
|  | Check to ensure all stiffeners are installed. |  |  |  |  |  |  |  |
|  | Shims installed as required and acceptable. Minimal number of shims to be used. (e.g. 20mm adjustment is shimmed with single 20mm shim, not 2 x 10mm shims) |  |  |  |  |  |  |  |
|  | Anchor bolts checked and accepted. |  |  |  |  |  |  |  |
|  | All structural bolts and bolted connections meet project specifications, checked and accepted. (S1.014 completed) |  |  |  |  |  |  |  |
|  | Welding & NDT of field welds completed and accepted (where required) NDT and Weld map reviewed. |  |  |  |  |  |  |  |
|  | Carry and return idler frames installed in the correct locations, surveyed and bolts checked. |  |  |  |  |  |  |  |
|  | Miscellaneous brackets and supports complete and acceptable. |  |  |  |  |  |  |  |
|  | Removal of temporary braces/guy wires completed and accepted. (after structural integrity achieved) |  |  |  |  |  |  |  |
|  | Protective coatings applied to suit SS-P118 and RVS-0000-SPE-S-003 and all touch-up painting completed. |  |  |  |  |  |  |  |
|  | Base-plates grouted and accepted. (S1.016 completed) |  |  |  |  |  |  |  |
|  | Survey Jig, free of damage and/or distortion, correctly seated in conveyor module. |  |  |  |  |  |  |  |
|  | Jacking Lock Nuts correctly torqued and marked. |  |  |  |  |  |  |  |
|  | Holt-Down Nuts correctly torqued and marked. |  |  |  |  |  |  |  |
|  | Protective coating of welds / drilled holes applied. |  |  |  |  |  |  |  |
|  | All idler rollers are seated to full depth on frame slots. |  |  |  |  |  |  |  |
|  | Foundation Sleeper Survey Reports completed and witnessed. |  |  |  |  |  |  |  |
|  | Verticality and level survey completed. |  |  |  |  |  |  |  |
|  | All idlers clocked, surveyed and accepted. |  |  |  |  |  |  |  |
|  | Module In-situ Survey completed and accepted. |  |  |  |  |  |  |  |
| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |  |  |  |  |
|  |  | Item | Initial | N/A | Comments |  |  |  |
| 1 |  | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |  |  |  |
| 2 |  | All Punchlist items noted on a separate Punchlist form and submitted for. |  |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |