| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| GRATING & GRIDMESH INSTALLATION |  | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm all grating and grid mesh panels are installed as per drawings and specifications and Load Bars are running in the correct direction. All installations to comply with MA-0000-C-1002, MA-0000-S-1011 and AS1657. |  |  |  |
|  | Confirm all grating panel edges are positioned over supporting members with a minimum contact of 30mm as per MA-0000-S-1011. |  |  |  |
|  | Confirm no grid mesh exceeds the maximum allowable cantilever of 250mm from a support to a permanent structure, as per MA-0000-S-1011. |  |  |  |
|  | Confirm the maximum horizontal gap between all grating panels does not exceed 10 mm at any point. |  |  |  |
|  | Confirm kick plates and pipe penetration bandings are installed as per design drawings, manufacturers specifications and MA-0000-S-1011. |  |  |  |
|  | Confirm all panel load bars are securely and fully welded to floor penetration collars. |  |  |  |
|  | Confirm all removable handrail sleeves comply with MA-0000-S-1011. |  |  |  |
|  | Confirm all grating panels are fastened using HILTl direct fastening system comprised of X-BT -GR M8/7 SN8 stainless steel studs and ANO X-FCM-M_L grating fastenings, installed at 1000mm nominal centres with a minimum of four fasteners per panel as per MA-0000-C-1002. To be installed as per manufacturers specifications. |  |  |  |
|  | Confirm any removable section of grid mesh has safety signage attached indicating the weight of the individual floor panel. |  |  |  |
|  | Confirm all surface treatment touch-ups have been completed. |  |  |  |
|  | Confirm all drill swarf, dust and general waste is removed. |  |  |  |
|  | Confirm area being inspected is safe for access. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |