| Tag No. «Name» - «Description» |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |  |  |  |  |  |  |  |
| Drawing No. |  |  | «DocumentList» |  |  |  | Task No. |  |  | «TaskName» |  |
| Additional Information: «Comments» |  |  |  |  |  |  |  |  |  |  |  |
| Item | Description |  |  |  |  |  |  | OK | N/A |  | Punch |
| PRE-PLACEMENT INSPECTIONS |  |  |  |  |  |  |  |  |  |  |  |
|  | Verify equipment has been aligned and levelled. |  |  |  |  |  |  |  |  |  |  |
|  | Confirm if any over scabbling has been repaired. |  |  |  |  |  |  |  |  |  |  |
|  | Surface is prepared for grout (scabbled, water conditioned etc.) |  |  |  |  |  |  |  |  |  |  |
|  | Sleeves are clean and clear. |  |  |  |  |  |  |  |  |  |  |
|  | Displaced air can be vented. |  |  |  |  |  |  |  |  |  |  |
|  | Formwork is properly waxed. |  |  |  |  |  |  |  |  |  |  |
|  | Formwork is sealed. |  |  |  |  |  |  |  |  |  |  |
|  | Curing products/facilities available. |  |  |  |  |  |  |  |  |  |  |
|  | Facilities for airing are available. |  |  |  |  |  |  |  |  |  |  |
|  | Pre pour inspection has been completed. |  |  |  |  |  |  |  |  |  |  |
|  | Confirm all grouting works comply with RVS-0000-SPE-C-003, RVS-0000-SPE-004, SS-C110 and the relevant Australian and vendor standards. |  |  |  |  |  |  |  |  |  |  |
| MIXING AND FINISHING INSPECTIONS |  |  |  |  |  |  |  |  |  |  |  |
|  | Grout Tube compressive strength test completed and accepted for batch material supplied with test sheet attached. |  |  |  |  |  |  |  |  |  |  |
|  | Grout mixing in accordance with manufacturer’s recommendations and the method statement is complete with daily records and sufficient quantity of test blocks available. |  |  |  |  |  |  |  |  |  |  |
|  | Flow cone testing acceptable. |  |  |  |  |  |  |  |  |  |  |
|  | Grout temperature in accordance with specification. |  |  |  |  |  |  |  |  |  |  |
|  | Grouting is complete and inspected with formwork removed & cured. |  |  |  |  |  |  |  |  |  |  |
|  | Grout is in full contact and is crack free. |  |  |  |  |  |  |  |  |  |  |
|  | Base plate has been Hammer sound tested for voids. |  |  |  |  |  |  |  |  |  |  |
|  | Grout is apparent in breather holes. |  |  |  |  |  |  |  |  |  |  |
|  | Grout type for anchor bolts correct. |  |  |  |  |  |  |  |  |  |  |
|  | Grout type for base plate correct. |  |  |  |  |  |  |  |  |  |  |
|  | Anchor bolts grouted first and cured. |  |  |  |  |  |  |  |  |  |  |
|  | Ambient temperature is within range as per manufacturers recommendation and SS-C110. |  |  |  |  |  |  |  |  |  |  |
| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Item |  | Initial | N/A | Comments |  |  |  |  |  |
| 1 |  | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |  |  |  |  |  |  |
| 2 |  | All Punchlist items noted on a separate Punchlist form and submitted for. |  |  |  |  |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |