| Tag No. «Name» - «Description» |  | Task No. «TaskName» |
| --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |
| Drawing No. | «DocumentList» |  |
| Comments | «Comments» |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm all handrails are fabricated and installed and in place as per drawings and specifications MA-0000-S-1015 and MA-0000-S-1016. |  |  |  |
|  | Confirm all kick plates are welded/bolted to vertical stanchion posts and kick plates are straight. (bolting preferred) |  |  |  |
|  | Confirm all vertical stanchion posts are plumb and level, equally spaced and at maximum centres of 2000mm. |  |  |  |
|  | Confirm all top rails are parallel, straight, level and 1017 mm from top of floor to centre axis of top rail. |  |  |  |
|  | Confirm all mid rails are parallel, straight, level and 560 mm from top of floor to centre axis of mid rail. |  |  |  |
|  | Confirm all handrail returns are welded and ground smooth. |  |  |  |
|  | Confirm handrail extensions are in place and fitted. |  |  |  |
|  | Confirm all bolts are tightened and installed with correct washers and nuts marked to indicate bolts have been correctly torqued. (S1-014 complete) |  |  |  |
|  | Confirm handrail stairs weld splices are ground smooth without concaves. |  |  |  |
|  | Confirm handrail and kneerail splice inserts are installed. |  |  |  |
|  | Confirm handrail splice joints are full penetration welds. |  |  |  |
|  | Confirm the gap between returns are within the range of Min 25 mm to Max 50 mm. |  |  |  |
|  | Confirm the gap between the handrail and the structure is a Min 25 mm to Max 50 mm. |  |  |  |
|  | Confirm the internal handrail on the stair is continuous. |  |  |  |
|  | Confirm the handrail is continuous for lead into the stair. |  |  |  |
|  | Confirm self-closing gates and drop bars are fabricated and installed in accordance with the drawings, specifications and scope of work. Confirm that gates and drop bars self-close. |  |  |  |
|  | Confirm surface treatments are in accordance with project specifications and SS-P118, including touch up painting. |  |  |  |

| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Description | Initial | N/A | Comments |
|  | All site modifications correctly marked up on master drawings as per project procedures and redline markup attached. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |