| Tag No: «Name» - «Description» |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Drawing No. «DocumentList» |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional Information: «Comments» |  |  |  |  |  |  |  |  |  |  |  | Task No. «TaskName» |  |  |  |
| Item | Description |  |  |  |  |  |  |  |  |  |  |  | OK | N/A | Punch |
|  | Record and confirm Module ID No________________________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Module survey data available from the module manufacturer & confirmed to match foundations Survey results. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Levelling plates installed and Surveyed to confirmed correct elevation / position prior to grouting. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Module installed in the specified position and confirmed to match survey results for mechanical centrelines. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Module in contact with Base plates as per specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Shims installed as required and acceptable. (e.g. 20mm adjustment is shimmed with single 20mm shim, not 2 x 10mm shims) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | All landing plates grouted and nuts torqued prior to welding. and attached |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Module to trestle or module to module bolted/welded connections acceptable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Module base pates welded in position and acceptable, NDT and weld maps completed and reviewed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Protective coating of module and on site welds completed as per SS-P118. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm that all remaining temporary shipping bracing has been removed post module installation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Module in situ survey completed and accepted (record readings below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Survey report complete and attached. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MECHANICAL CENTRELINE |  |  |  | SURVEY DATA |  |  |  |  |  |  | Remarks |  |  |  |  |
|  |  |  |  | Elevation |  | East |  |  | North |  |  |  |  |  |  |
| Grid Ref. |  |  | Mark Number | Design | Actual |  |  |  |  |  |  |  |  |  |  |
| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Item |  |  |  |  | Initial | N/A |  | Comments |  |  |  |  |  |
| 1 |  | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  | All Punchlist items noted on a separate Punchlist form and submitted for entry into CCMS. |  |  |  |  |  |  |  |  |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |