| Tag No. «Name» - «Description» |  |  |  |  |  |  | Task No. «TaskName» |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |  |  |  |  |  |  |
| Drawing No. |  |  | «DocumentList» |  |  |  |  |  |  |  |
| Additional Information: «Comments» |  |  |  |  |  |  |  |  |  |  |
| Item | Descriptio |  |  |  |  |  |  | OK | N/A | Punch |
|  | Foundations as per AFC drawings & verified by survey |  |  |  |  |  |  |  |  |  |
| INSPECTION OF STAIRWAYS |  |  |  |  |  |  |  |  |  |  |
|  | Non-slip nosing installed to top stair tread as per MA-0000-S-1012. |  |  |  |  |  |  |  |  |  |
|  | Step rise is 200mm (preferred) |  |  |  |  |  |  |  |  |  |
|  | Tread depth is 230mm (preferred) |  |  |  |  |  |  |  |  |  |
|  | Kickplates installed as per MA-0000-S-1012. |  |  |  |  |  |  |  |  |  |
|  | Diagonal bracing installed for all spans exceeding 5 metres as per MA-0000-S-1012. |  |  |  |  |  |  |  |  |  |
|  | Top and bottom landing platform level with surrounding structure. |  |  |  |  |  |  |  |  |  |
|  | Kickplate & mesh panels installed correctly as per AFC Drawings & Std Drawing MA-0000-S-1012 and MA-0000-C-1002. |  |  |  |  |  |  |  |  |  |
|  | All stair bolts have been installed, tensioned and accepted and S1-014 completed. |  |  |  |  |  |  |  |  |  |
|  | Base of stair stringer has been grouted and bolted as per MA-0000-S-1012. |  |  |  |  |  |  |  |  |  |
|  | Selection of access complies with AS1657, figure 2.1. |  |  |  |  |  |  |  |  |  |
| INSPECTION OF LADDER ACCESS |  |  |  |  |  |  |  |  |  |  |
|  | Ladder installation complies with MA-0000-S-1013. |  |  |  |  |  |  |  |  |  |
|  | Ladder width 400mm across. |  |  |  |  |  |  |  |  |  |
|  | Ladder rungs installed equally spaced at 250mm (minimum) and 300mm (maximum). |  |  |  |  |  |  |  |  |  |
|  | Top of rung is level with any landing used for access. |  |  |  |  |  |  |  |  |  |
|  | Ladder rungs are 20mm round bar with anti slip covers to manufacturers specification as per MA-0000-S-1013. |  |  |  |  |  |  |  |  |  |
|  | All cage hoops installed and comply with MA-0000-S-1013. |  |  |  |  |  |  |  |  |  |
|  | Cage hoops and ladder rungs extend past accessways as per MA-0000-S-1013. |  |  |  |  |  |  |  |  |  |
|  | Abrasive nosing installed to grating at top of ladder as per MA-0000-S-1013. |  |  |  |  |  |  |  |  |  |
|  | Selection of access complies with AS1657, figure 2.1. |  |  |  |  |  |  |  |  |  |
|  | All installations comply with AS1657. |  |  |  |  |  |  |  |  |  |
| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |  |  |  |  |  |  |
|  |  | Item |  | Initial | N/A | Comments |  |  |  |  |
| 1 |  | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |  |  |  |  |  |
| 2 |  | All Punchlist items noted on a separate Punchlist form and submitted for entry into CCMS. |  |  |  |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |