| Tag No. «Name» - «Description» |  |  |
| --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |
| Drawing No. | «DocumentList» | Task No. «TaskName» |
| Additional Information: | «Comments» |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
| - ‘ | Foundations as per AFC drawings & verified by survey. |  |  |  |
|  | All members correct as per AFC drawings. |  |  |  |
|  | Steelwork installed correctly as per AFC Drawings. |  |  |  |
|  | Bolted connections checked for correct sizing, finish, grade, nut/ washer combination & tension complete with appropriate markings as specified in AFC Drawings / MA-0000-S-1002 / AS4100. (S1-014 complete) |  |  |  |
|  | Lock nuts installed and torqued correctly to procedure. |  |  |  |
|  | Isolation sleeves installed as specified between dissimilar metals. |  |  |  |
|  | All welded joints inspected, NDT undertaken as per AFC Drawings/ MA-0000-S-1017 / AS/NZ 1554.1. |  |  |  |
|  | Shims/packing of correct type & sizing installed where specified and as per MA-0000-M-1007. |  |  |  |
|  | Verticality/ alignment & RL survey conducted & results comply with AFC Drawings/ AS 4100- attach results. |  |  |  |
|  | Grouting complete & complies with RVS-0000-SPE-C-003, RVS-0000-SPE-004 and the relevant Australian and vendor standards. (S1-016 complete) |  |  |  |
|  | All earthing complete. |  |  |  |
|  | All expansion joints installed correctly. |  |  |  |
|  | Sliding joints installed correctly and mating steelwork appropriately prepared as per vendor requirements. |  |  |  |
|  | All stairways installed correctly as per AFC Drawings & Std Drawing MA-0000-S-1012. |  |  |  |
|  | Access ladders, safety cages & gates installed correctly as per AFC Drawings & Std Drawing MA-0000-S-1013. |  |  |  |
|  | Floor plate/ grid mesh & fixings installed correctly as per AFC Drawings & Std Drawing MA-0000-S-1011. |  |  |  |
|  | Lifting lugs comply with AFC Drawings/ Std Drawing MA-0000-M-1124 & NDT has been completed. |  |  |  |
|  | Hand railing, kickplate & mesh panels installed correctly as per AFC Drawings & Std Drawing MA-0000-S-1015/MA-0000-S-1016. |  |  |  |
|  | All surface treatments inspected & comply with SS-P118. |  |  |  |
|  | Any steelwork deemed hazardous to be painted yellow with diagonal black stripes as per AS1318. |  |  |  |
|  | Safety signage installed as per AS1319, project specifications and design drawings. |  |  |  |
|  | Structure & surrounding area clean & clear from all debris. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |